FABRIC CONSTRUCTION CHART NOTES

FABRIC CONSTRUCTION

- fibers are created into yarns

- yarns are then woven or knit into fabrics

- fabrics are used to create projects

# BLENDED YARNS, THREADS, AND FABRICS

- blends were created to utilize the positive characteristics of each fiber

- different fibers that have been combined into one fabric, usually one natural and one synthetic

- EXAMPLE:  polyester/cotton blend- the original cotton characteristics are improved and the fabric becomes more wrinkle resistant, stronger, and mildew resistant

WOVENS

- a woven fabric consists of warp and weft yarns

- weaving occurs when two or more yarns are woven together at right angles to make a fabric

- strong and easy to sew on

- suggested for beginners to use

GRAIN (LINE)

- selvage - parallel to lengthwise grain; the tightly woven edges of the fabric

- lengthwise - parallel to the selvage; stronger threads (warp threads)

- crosswise - at right angle to the lengthwise threads; perpendicular to the selvage (weft threads)

- bias - diagonal angle; runs at a 45-degree angle to the selvage edge; provides stretch

KNIT

- knitting- yarns are fashioned by needles into a series of interlocking loops to make a fabric

- knits provide stretch

- knits don't need a seam or edge finish; will not fray

- ball point needle is recommended for sewing on knits

- single knit fabric curls to the right side when stretched

- interlock knit is thicker than single knits and when stretched they don't curl

NON-WOVEN/ FELTING

- made when fibers are pressed together using heat and moisture

- examples include: felt and non-woven interfacing

NAP

- short fibers that create texture on fabric

- use a nap layout

- treat the same way as a one-way directional fabric